# Thermodynamic Failure Mechanism of the Shipping Failure Mode

This subsection is formatted as a standalone insert for an engineering report. It is written to make the failure mechanism explicit in first-principles thermodynamic terms.

## Scope and model statement

The shipping problem is fundamentally a gas-inventory and compliance-volume problem in a distributed, manifolded nitrogen enclosure. The relevant internal gas space is the aggregate of the closed aluminum optical modules, the intake and exhaust manifolds, the PFA interconnect tubing, and the shipping-time compliance bag. The engineering requirement is not merely to avoid peak overpressure. The requirement is to maintain the total connected nitrogen volume at nonnegative, and preferably slightly positive, pressure relative to ambient throughout the coupled shipping pressure–temperature envelope. If the internal pressure becomes negative with respect to ambient at any point in the cycle, then any of the existing seal and joint locations can become an inward leakage path for atmospheric contamination.

For a first-principles explanation of the failure mechanism, the system is idealized as follows.

- The manifolded optical assembly is represented by one lumped rigid gas volume, Vfixed.

- The foil bag is represented as a geometrically compliant but not elastically compliant volume, Vbag, bounded by hard limits 0≤Vbag≤Vbag,max.

- The bag is assumed to have negligible stiffness over its usable range, so while it is neither fully collapsed nor fully expanded, the internal pressure can track ambient with negligible pressure penalty.

- The outward-only shipping valve is assumed to be ideal for the purpose of mechanism proof: it opens only outward, closes perfectly, and never leaks inward.

- Nitrogen is treated as an ideal gas over the relevant pressure and temperature range.

This idealization is intentionally conservative with respect to the failure mechanism argument. If the system is shown to go sub-atmospheric even with an ideal outward-only valve and an ideal zero-stiffness bag, then the mechanism does not depend on valve back-leakage or bag elasticity assumptions. It follows directly from conservation of gas mass and the finite available compliance volume.

## Notation

P = absolute pressure [Pa, bar abs]

Pint = internal absolute pressure of the connected nitrogen system [Pa, bar abs]

Pamb = ambient absolute pressure [Pa, bar abs]

Pbias = desired minimum positive pressure bias above ambient [Pa, bar]

T = absolute gas temperature [K]

n = nitrogen mole inventory retained within the system [mol]

R = universal gas constant [J mol​−1 K​−1]

Vfixed = total rigid connected nitrogen volume of modules, manifolds, and tubing [m​3, L]

Vbag = instantaneous shipping-bag volume [m​3, L]

Vbag,max = maximum available bag volume [m​3, L]

Vbag,0 = bag volume at the initial seal-up state [m​3, L]

Ph* = internal absolute pressure at the outbound high-altitude/high-temperature state after any venting [Pa, bar abs]

T0 = initial seal-up temperature [K]

Th = outbound high-temperature state [K]

Tr = return-state temperature [K]

α = outbound expansion factor from the initial state to the outbound hot/low-pressure state [dimensionless]

γ = return-leg underpressure factor used in the post-vent threshold expression [dimensionless]

## Governing relations

The governing constitutive law is the ideal-gas relation.

Equation (1). Ideal-gas relation

PV=nRT

For the present lumped model, the total gas volume is the sum of the rigid connected volume and the bag volume.

Equation (2). Total connected gas volume

Vtot=Vfixed+Vbag

The bag is assumed to be nonelastic and bounded only by geometric end stops.

Equation (3). Compliance bounds

0≤Vbag≤Vbag,max

The essential feature of the problem is that the shipping cycle can change not only P and T, but also n. Once nitrogen is vented on the outbound leg, the return state is no longer described by a fixed-mass gas process referenced to the initial seal-up state. The system has undergone an irreversible reduction in retained gas inventory.

## Regime I — bag active, internal pressure tracks ambient

While the bag remains within its geometric range, it acts as a zero-stiffness compliance reservoir. In that regime the internal pressure is approximately ambient, or ambient plus a deliberately imposed small positive bias.

Equation (4). Internal pressure while compliance remains available

Pint≈Pamb+Pbias

If a specific internal pressure target is prescribed, the total gas volume required to support that pressure at a given state is obtained directly from Equation (1).

Equation (5). Required total volume at a target state

Vreq=nRTPint,target

and the corresponding bag volume requirement is

Equation (6). Required bag volume

Vbag,req=Vreq−Vfixed

As long as 0≤Vbag,req≤Vbag,max, the bag can absorb the thermodynamic volume excursion without forcing either overpressure or underpressure. In this regime the design is behaving ideally.

## Regime II — outbound saturation of the bag and loss of nitrogen inventory

Consider an initial seal-up state 0 and an outbound hot, low-ambient-pressure state h. If no venting occurs between those two states, then the mole inventory remains constant and Equation (1) gives

Equation (7). No-vent relation between the initial state and outbound state

P0Vfixed+Vbag,0T0=PhVfixed+Vbag,hTh

Define the outbound expansion factor

Equation (8). Outbound expansion factor

α=P0Ph ThT0

Then the no-vent requirement at the outbound state is

Equation (9). Outbound no-vent criterion

αVfixed+Vbag,0≤Vfixed+Vbag,max

which can be rearranged into an explicit upper bound on the rigid connected system volume:

Equation (10). Maximum rigid connected volume that avoids outbound venting

Vfixed≤Vbag,max−αVbag,0α−1  α>1

Equation (10) is one of the most useful screening relations in the analysis. It shows that the apparent adequacy of a given bag depends not only on its maximum volume, but also on the initial fill state and the actual pressure–temperature excursion. Once Equation (10) is violated, the bag must either saturate and permit overpressure to develop, or the outward-only valve must vent nitrogen.

If the bag saturates and the outward-only valve vents to a pressure Ph* at the outbound state, then the retained nitrogen inventory is

Equation (11). Retained nitrogen inventory after outbound venting

nretained=Ph*Vfixed+Vbag,maxRTh

This expression is the central statement of the irreversible loss mechanism. The outbound event fixes a reduced mole inventory. That loss cannot be undone during the return leg by an ideal outward-only valve.

## Regime III — return leg, bag collapse, and onset of sub-atmospheric pressure

On the return leg, ambient pressure rises and the gas cools. The bag therefore contracts. If the bag reaches Vbag=0, then the entire retained gas inventory must occupy the rigid connected volume only. The return-state internal pressure is then

Equation (12). Return-state pressure after prior venting and complete bag collapse

Pr,int=nretainedRTrVfixed=Ph* Vfixed+Vbag,maxVfixed TrTh

Equation (12) shows the controlling dependence directly. Return pressure improves with larger compliance volume and worsens with larger rigid connected volume. Once the bag has collapsed, the bag is no longer a pressure-protective device. The problem is purely a question of whether enough nitrogen mass remains to fill Vfixed at the required return condition.

If the acceptance criterion is merely to avoid sub-atmospheric pressure, then the requirement is

Equation (13). Nonnegative-gauge requirement at the return state

Pr,int≥Pamb,r

If the requirement is to retain a positive seating margin on the existing seals, then the stronger condition is

Equation (14). Positive-bias requirement at the return state

Pr,int≥Pamb,r+Pbias

Substituting Equation (12) into Equation (14), define

Equation (15). Return-leg underpressure factor

γ=Pamb,r+PbiasThPh*Tr

Then the maximum rigid connected volume consistent with the required return-state pressure is

Equation (16). Maximum rigid connected volume that avoids return-leg underpressure after venting

Vfixed≤Vbag,maxγ−1  γ>1

Equation (16) is the corresponding post-vent screening relation. If the actual rigid connected volume exceeds this value, then return-leg sub-atmospheric pressure is unavoidable after the bag has collapsed, even if the valve is perfectly one-way and never leaks inward.

## Physical interpretation

During ascent and thermal exposure, the gas attempts to expand because T increases and Pamb decreases. If the available compliance volume is sufficient, the bag accommodates that expansion and the internal pressure remains close to ambient. If the available compliance is insufficient, the bag saturates. The system then either develops overpressure or vents nitrogen outward. Once nitrogen is vented, the retained gas inventory is reduced from n0 to nretained.

On the return leg, the system is no longer a fixed-mass gas volume referenced to the original seal-up condition. It is a depleted-mass gas volume. As ambient pressure rises and temperature falls, the bag contracts until it eventually collapses. At that point the remaining nitrogen must occupy only Vfixed. If Equation (14) cannot be satisfied, then the internal pressure falls below the required return-state pressure; if Equation (13) is violated, the internal pressure becomes sub-atmospheric with respect to ambient. Under those conditions, the many existing seals and joints of the manifolded optical assembly become the equalization path. Ambient air ingress is therefore not a separate or speculative mechanism. It is the thermodynamically favored consequence of a depleted nitrogen inventory combined with insufficient residual compliance.

The high-pressure side of the cycle is therefore not the principal contamination risk. Outbound overpressure is bounded by available compliance and by the relief path. The critical contamination risk is the low-pressure side of the cycle: irreversible loss of nitrogen during the outbound leg, followed by insufficient remaining nitrogen mass during descent and cooling.

## Engineering consequence for the present shipping concept

The present foil-bag concept should therefore be evaluated against Equations (10) and (16), not only against a peak overpressure argument. A bag that appears acceptable when judged solely by its ability to buffer outbound expansion may still be fundamentally inadequate if it cannot preserve enough retained nitrogen inventory to keep the rigid connected system nonnegative relative to ambient on the return leg. This is why the critical unknown in the present hardware is the true value of Vfixed: the total rigid nitrogen volume of the interconnected optical modules, tubing, and manifolds.

For the current concept, the bag volume is known to be finite and the bag is not elastically restorative. It provides only temporary geometric compliance. Consequently, once the bag has either fully expanded or fully collapsed, the system response is governed directly by Equations (11) through (16). The engineering design question is therefore not simply whether a bag is present, but whether the available compliance volume and retained gas inventory are sufficient to guarantee that the connected nitrogen space never crosses below the required return-state pressure. If that cannot be guaranteed, then contamination ingress through the pre-existing distributed seal network is a predictable outcome of the transport thermodynamics rather than an anomalous field event.

## Recommended concluding sentence

The critical shipping failure mode is not outbound overpressure but return-leg underpressure following irreversible nitrogen loss. Once the passive bag saturates and nitrogen is vented, the retained gas inventory may be insufficient to refill the rigid connected system volume after the bag collapses during descent and cooling. If the resulting internal pressure falls below ambient, the distributed seal network becomes an ingress path for atmospheric contamination by first principles.